# D19 WP Test BUGS

# MICRO

# Data pre processing

- Land use categories

Data collected through KVR services have to be transformed in order to be used by all the partners. The Land Use Data Base provided is very detailed and huge. The categories of the various land use all over the KVH area are numerous and not automatically usable. It has been necessary to harmonise the date categories according partners needs: for instance the LAICA and Corine Land Cover tables categories for Vito’s modelling purposes and the transformation of ground characteristics for AAC models.

The BUGS project relies on an integration of models (Figure 1) dealing with various spatial scales (macro and micro scale) that have to be articulated. The inputs from planners are incorporated to simulate urban structure changes that might affect the atmospheric pollutant release and dispersion. Such integration of models requires peculiar attention to the data flows and the consistency of the information to be delivered to each partner. The previous GEOMAP work-package was in charge to deliver Land Use - Land Cover (LU-LC) data for partners, derived from imagery remote sensing or data -bases.

Figure 1 Data-flow

SoundPLAN

AUTO(CDV)

ARPS –AURORA (VITO)

ENVI-Met

AUTO (CDV)

Land Cover-Land Use

AUTO is used at both scales

Our objectives here were to provide several layers from remote sensing data (LU-LC, temperature, and vegetation index at various spatial resolutions) and data - bases. It appears rapidly that the information required by all the partners were not totally relevant (precision, availability, format, resolution, type of data etc.) or quite different according semantic contents or scales. Several data-bases have been used to satisfy the requirements. One of the principal problems was to get precise definitions of the partners needs in order to be able to provide reliable and relevant data.

A general land cover type like the LAICA categories proposed by our Belgium partner and needed at the macro level, could not satisfy the micro level neither does the Corine Landcover database. The Corine Landcover areas are too coarse for this level thus another database more precise has to be found. In a general case study application, the initial needed coverage might be obtained through remote sensing data (LandsatTM images) and/or vector files provided by KVR. This is the reason why a careful stepwise methodology has to be set up.

| MICRO | Provide spatial information for the micro zones Mulheim and Oberhausen. - Land use – land cover layers:extraction of microzones and aggregation in main categories - Buildings contours and Height of the buildings in Oberhausen test- zone - Building height in Mülheim - Noise regulation map with land use – land cover categories (with ground /soil compactness maps) |
| --- | --- |

Objectives

The objectives of this work package are to provide the MICRO scale partners with spatial information usable in the models they use. The selected areas are 2 derelict areas in Mülheim and areas in Oberhausen for which Land-Use and Land Cover (LU-LC) are required. Even if partners agree on the general categories of LU-LC, they still have various requirements when they ask for specific types adapted to their model or relevant for their objectives. For instance the file needed by the TRAFFIC model is constrained by the number of total zones that can be introduced in the model (650) at the MACRO level. For the SoundPLAN model other requirements need to be satisfied concerning the ground quality. The use of new Nordic method to model sound propagation leads to fix a maximum of 7 ground types, that are associated to the degree of compactness: from acoustically very soft like snow to hard surfaces as concrete or water.

Methodology and scientific achievements related to the WP

A link has to be strongly defined between databases used in order to ensure coherence within the data flows between the models and the results. Each class of the used databases has been fixed in collaboration with the DTPI and detailed in order to ensure a general understanding of class contents by all partners (metadata). The table 1 presents the categories of three data – bases used, and the link between each of them.

Among the data–bases gathered in the BUGS project, the available KVR Land Use-Land Cover (LU-LC) data- base which was ought to be used for all scales, but it turned out to be far too detailed. This introduces two types of problems: first a too large amount of classes and of polygons over the KVR area; second the difficulty to reproduce the analysis over another city which may not have this type of data – base. Confronted with to the amount of polygons and the large range of thematic classes of the KVR LU-LC layers, an adapted methodology in several steps was set up using conversion GIS capacity from raster to vector data models, and filtering processing over the KRV area.

Table 1 Extraction of the conversion keys tables

| Laica ID | Laica categories | Corine ID | Corine type (combined) | # KVR types | KVR ID |
| --- | --- | --- | --- | --- | --- |
| 1 | urban | 1 | urban fabric | Residential areas, 1-3 floors Residential areas, 1-5 floors Residential areas, more than 5 floors Built up areas, mixed use Service areas Residential and connection streets Pedestrian zones Parking lots Other public squares | 10 20 30 40 80 150 160 170 200 |
| 3 | industry | 3 | Industrial / commercial units | Industrial areas (including mining) Business areas Farming buildings Energy supply Water supply Waste water disposal Waste incineration, recycling plant Other built up areas | 60 50 90 210 220 230 241 100 |
| 3 | industry | 4 | Road / rail networks | Motorways and alike Highways Railways and belonging systems | 110 140 180 |
| 3 | industry | 6 | Airports | Airports | 190 |
| 6 | grass | 7 | Mineral extraction sites | Earth removal areas | 260 |
| 6 | grass | 8 | Dump sites | Waste disposal sites Storing areas for earth and waste earth Slag hills | 240 250 500 |
| 6 | grass | 9 | Construction sites | Fallow, set-aside land / residential areas Fallow, set-aside land / business and industrial areas Fallow, set-aside land / mining areas Traffic areas out of use | 450 460 480 490 |
| 10 | woodland | 10 | Green urban areas | Public and private green areas and parks Churchyard Allotment gardens Green gap areas | 270 280 290 320 |

Two of the models need the population information associated to residential areas: for AUTO it is needed to derive the changes in transportation flux, and for SoundPLAN to calculate the population affected by the noise. So the demographic sectors have also to be taken into consideration. The same categories will be used by ENVI-Met for decision of covered versus uncovered ground that introduce modifications in the model (water is an extra category for this last model for instance).

Thus the transformations of the LU-LC layers have to fit numerous constrains: semantic coherence and relevance, geometric reliability, and specific modeling restrictions.

The transformation from the KVR types to these new categories has been realized carefully to provide relevant classes to the noise assessment modeling (table 2).

| ## Catsoil_ID | ## Soil compactness | # KVR categories |
| --- | --- | --- |
| Cat. G | Hard surfaces | Residential areas, 1-3 floors Residential areas, 1-5 floors Residential areas, more than 5 floors Built up areas, mixed use Business areas (buildings and sealed areas) Industrial areas (buildings and plants) Sport and recreation areas Service areas Farming buildings Other built up areas (military, buildings and plants) Residential and connection streets Pedestrian zones Parking lots (buldings) Railways and belonging systems (buildings and sealed areas) Energy supply (buildings) Water supply (except 223) Waste water disposal Waste incineration, recycling plant (buildings and plants) Earth removal areas (buildings) Churchyard (buildings) Airports (except 193) Motorways and alike Highways Water courses Canals and harbours (i.e. the water in the harbour) Lakes and ponds Market garden. Nursery (buildings) Fallow, set-aside land / mining areas (buildings and plants) |
| Cat. F | compacted dense ground | Business areas (except buildings and plants) Industrial areas (except buildings and plants) Other built up areas (military, other areas) Parking lots (except buildings) Railways and belonging systems (rails and others, non built up) Airports (other areas, non built up) Other public squares Energy supply (except buildings) Fallow, set-aside land / residential areas Fallow, set-aside land / business and industrial areas Traffic areas out of use |
| Cat. E | compacted field and gravel | Earth removal areas (except buildings) Waste disposal sites (except buildings and plants) Storing areas for earth and waste earth Public and private green areas and parks Allotment gardens Churchyard (except buildings) Grounds for game and sport Camping areas Fallow, set-aside land / mining areas Slag hills |
| Cat. D | normal uncompacted ground | Water supply (other areas, ex. pasture) Green gap areas (outside of city) Permanent meadows and pastures Farm land, arable land Market garden. Nursery (except buildings) Stands, smaller areas vegetated or with growth of trees Areas for reforestation Farming and forestry areas out of use |
| Forest | Forest dec | Deciduous forest |
|  | Forest con | Coniferous forest |
|  | Forest mix | Mixed forest |

Table 2 Noise assessment needs

Among the results obtained during this part of the program, it was important for us to be able to provide relevant and reliable data to all our partners. Classification of LU-LC results lead to specific information layers, adapted to partner’s requirements. Apart from peculiar data file format, this work-package reveals the real necessity to have a consistent dataflow chart in order to avoid troubles when dealing with integrated programs.

- Surface digital model

Digital surface model have been necessary to approximate houses height in KVR selected areas (Mülheim and Oberhausen). For Mülheim the house’s height data were provided through a specific data-base with polygons and heights referend to the sea leval. A Digital Terrain Model (DTM) in a grid format was provided by KVR. For  Oberhausen it was necessary to create them by another way.

Mülheim: Using GIS capacities the height per buildings has been extracted. The digital terrain model (DTM) provided by KVR, and the transformed house’s heights layer (into a grid format too)  have been used in order to obtain (1) the maximum height per building data and the elevation mean per building data. A new layer was created substracting from the digital surface model (DSM) values the building heights (figure 2).

Oberhausen: A general file with houses polygons has been digitized. The heights have been approximated using the KVR Land Use Land Cover data base, using the same categories of built up areas (residential, industrial, etc.).

Sea level

Digital Terrain Model

Digital Surface Model

Oberhausen have been abandoned during the study.